CSE 4571 – PRACTICAL ROBOTICS PROJECTS WITH ARDUINO

EXPERIMENT 4
STEPPER MOTOR POSITION AND DIRECTION CONTROL USING A4988

| Duration | Level | Mode | CO Mapping |
| --- | --- | --- | --- |
| 2 Hours | B.Tech CSE Final Year | Hardware / Wokwi | CO2, CO3 |

1. Aim of the Experiment

To interface a bipolar stepper motor with an A4988 driver and control its position and direction.

2. Objectives

- Understand STEP/DIR control.

- Generate controlled step pulses.

- Demonstrate direction reversal.

- Relate steps to angular position.

3. Components / Wokwi Parts

| Component | Quantity | Purpose |
| --- | --- | --- |
| Arduino UNO | 1 | Microcontroller / pulse generation |
| Bipolar Stepper Motor | 1 | Discrete-position actuator |
| A4988 Driver | 1 | Stepper driver / STEP-DIR interface |
| Breadboard | 1 | Circuit assembly |
| Jumper wires | As required | Interconnections |

4. Theory

A stepper motor advances in discrete increments. The A4988 accepts STEP and DIR signals. Each STEP pulse advances the motor by a step, while DIR selects the direction. ENABLE controls whether the driver is enabled. Wokwi simulates the bipolar stepper and A4988 so that position and direction can be observed.

Number of steps × step angle → commanded angular displacement

5. Pre-Lab Questions

- What is a stepper motor?

- What do STEP and DIR mean?

- Why is a driver used?

6. Circuit Diagram

Figure 1. A4988 STEP/DIR interface with bipolar stepper motor.

7. Connection Table

| Signal / Component | Arduino Pin | Connection Details |
| --- | --- | --- |
| STEP | D3 | A4988 STEP input |
| DIR | D4 | A4988 DIR input |
| ENABLE | D5 | A4988 ENABLE input |
| A4988 VDD | 5V | Logic supply |
| A4988 GND | GND | Common ground |
| Motor outputs | 1A, 1B, 2A, 2B | Two bipolar motor coils |
| RESET | SLEEP |  |
| SLEEP | 5V |  |
| M1,M2,M3 (Shorted) | GND |  |

8. Circuit Diagram Implemented Using Hardware’s

9. Procedure

- Add the stepper and A4988.

- Connect logic supply and control lines exactly as shown.

- Connect the motor coils to the A4988 motor outputs.

- Run the program.

- Observe clockwise and counter-clockwise movement.

- Modify the step count and compare expected/observed angle.

- Record direction, step count and expected/observed angle.

10. Arduino Program

11. Observation Table

| Sl. No. | Direction | Steps | Expected Angle | Observed Angle |
| --- | --- | --- | --- | --- |
| 1 |  |  |  |  |
| 2 |  |  |  |  |
| 3 |  |  |  |  |
| 4 |  |  |  |  |
| 5 |  |  |  |  |

12. Expected Output / Analysis

The program commands 100 steps in one direction, waits one second, then commands 100 steps in the reverse direction. The motor therefore alternates between the two directions. The actual angular displacement depends on the step angle represented by the simulated motor configuration.

13. Result

The bipolar stepper motor was successfully interfaced with the A4988 driver in Wokwi. STEP/DIR control, direction reversal and the relationship between commanded steps and angular movement were verified.

14. Post-Lab Questions

- Command a 90° movement.

- Add a potentiometer to change step rate.

15. Precautions

- Verify pin assignments before running the simulation.

- Use common ground for connected devices.

- Record both input condition and observed output.

- Do not change STEP, DIR and ENABLE pins without updating the program.

- For physical implementation, consider current, voltage, motor loading, driver configuration and thermal requirements.

16. Suggested Extension

Modify the program to add one meaningful feature, document the change, and preserve the original behavior as a baseline.

17. Student Record

| Student Name | Registration No. | Date | Faculty Signature |
| --- | --- | --- | --- |