Date:

Aim of the Experiment:

Interfacing of LEDs with Arduino Uno.

Objective:

- Create an Arduino Sketch to control the intensity of one or more LEDs.

- SOS LED Signaling using Arduino

- Design a Knight Rider using Arduino

Components/Equipment Required:

| Sl No. | Name of the Component/Equipment | Specification | Quantity |
| --- | --- | --- | --- |
| 1 | Arduino UNO R3 | 16MHz | 1 |
| 2 | Arduino UNO cable | USB Type A to B | 1 |
| 3 | Resistors (carbon type) | ¼ watt (330Ω) or 220 Ω | 4 |
| 4 | LED | Red, Blue, Green, White | As per the requirement |
| 5 | Breadboard | 840 Tie points | 1 |
| 6 | Jumper Wire | ----------------------- | As per the requirement |

# Objective 1

Circuit/Schematic Diagram:

Code:

Observation:

# Objective 2

SOS LED Signalling using Arduino

Problem Statement:

Design and implement an SOS LED Signaling System using an Arduino Uno, one external LED, and a current-limiting resistor. The system should transmit the internationally recognized SOS distress signal in Morse code by controlling the LED with different ON and OFF durations.

The Morse code for SOS is represented as:

S → • • • (Three short flashes)

O → – – – (Three long flashes)

S → • • • (Three short flashes)

Requirements

1. Connect an external LED to Digital Pin 13 of the Arduino through a 220 Ω or 330 Ω current-limiting resistor.

2. Configure Pin 13 as an OUTPUT in the setup() function.

3. In the loop() function, generate the SOS signal using the following timing:

○ Dot (•):

- LED ON for 250 milliseconds

- LED OFF for 250 milliseconds

○ Dash (–):

- LED ON for 500 milliseconds

- LED OFF for 500 milliseconds

Expected Output

The LED should continuously blink in the following pattern:

••• ––– •••

where:

- Short blink (Dot) = 250 ms ON, 250 ms OFF

- Long blink (Dash) = 500 ms ON, 500 ms OFF

- Pause between successive SOS signals = 3000 ms

Deliverables:

- Circuit diagram.

- Arduino program (.ino file).

- Demonstration of the working SOS LED signaling system.

Circuit/Schematic Diagram:

Code:

Observation:

# Objective 3

Circuit/Schematic Diagram:

Code:

Observation:

Post Experiment Questionnaire:

- Draw a simple block diagram of an Embedded System.

- List out the important characteristics of embedded system.

- List out the different types of embedded system in a Tree structure.

- Biomedical surgery through robots is based on which type of embedded system?

- List out the possible different processors in Embedded development board modules.

- Differentiate between firmware and embedded software.

- All embedded software is firmware. (TRUE/FALSE). Justify your Answer.

- List out the different embedded programming languages used in Arduino sketch.

- Draw a neat and clean flow chart to get a final product in Embedded development life cycle.

- Differentiate between manual and automatic control system.

- List out the different types of Arduino boards available in the market and give one specific application of each board.

- What is PWM pins in Arduino? How many PWM pins are present in Arduino UNO?

- Can we give power supply to the Arduino Uno through USB connection and DC power barrel jack simultaneously? Justify your answer.

- Which voltage regulator is present in Arduino module?

- In the market many microcontrollers are available. Raspberry Pi is one of them. Distinguish the advantage and disadvantages of Arduino when it is compared with Raspberry Pi.

- How to minimize the number of pins required to connect more number of LEDs.

Name of the Student

Registration No

Branch, Section